Digital Electronics Question and Answers

Flip Flops-1

Topic 6

Flip Flops-1

- Latches

- Flip Flops

# Digital Electronics Question & Answers

Latches

1. A latch is an example of a ___________
a) Monostable multivibrator
b) Astable multivibrator
c) Bistable multivibrator
d) 555 timer
Answer: c
Explanation: A latch is an example of a bistable multivibrator. A Bistable multivibrator is one in which the circuit is stable in either of two states. It can be flipped from one state to the other state and vice-versa.

2. Latch is a device with ___________
a) One stable state
b) Two stable state
c) Three stable state
d) Infinite stable states
Answer: b
Explanation: Since a latch works on the principal of bistable multivibrator. A Bistable multivibrator is one in which the circuit is stable in either of two states. It can be flipped from one state to the other state and vice-versa. So a latch has two stable states.

3. Why latches are called memory devices?

a) It has capability to stare 8 bits of data
b) It has internal memory of 4 bit
c) It can store one bit of data
d) It can store infinite amount of data
Answer: c
Explanation: Latches can be memory devices, and can store one bit of data for as long as the device is powered. Once device is turned off, the memory gets refreshed.

4. Two stable states of latches are ___________
a) Astable & Monostable
b) Low input & high output
c) High output & low output
d) Low output & high input
Answer: c
Explanation: A latch has two stable states, following the principle of Bistable Multivibrator. There are two stable states of latches and these states are high-output and low-output.

5. How many types of latches are ___________
a) 4
b) 3
c) 2
d) 5
Answer: a
Explanation: There are four types of latches: SR latch, D latch, JK latch and T latch. D latch is a modified form of SR latch whereas, T latch is an advanced form of JK latch.

6. The full form of SR is ___________
a) System rated
b) Set reset
c) Set ready
d) Set Rated

Answer: b
Explanation: The full form of SR is set/reset. It is a type of latch having two stable states.

7. The SR latch consists of ___________
a) 1 input
b) 2 inputs
c) 3 inputs
d) 4 inputs
Answer: b
Explanation: SR or Set-Reset latch is the simplest type of bistable multivibrator having two stable states.
The diagram of SR latch is shown below:

8. The outputs of SR latch are ___________
a) x and y
b) a and b
c) s and r
d) q and q’

Answer: d
Explanation: SR or Set-Reset latch is the simplest type of bistable multivibrator having two stable states. The inputs of SR latch are s and r while outputs are q and q’. It is clear from the diagram:
.

9. The NAND latch works when both inputs are ___________
a) 1
b) 0
c) Inverted
d) Don’t cares
Answer: a
Explanation: The NAND latch works when both inputs are 1. Since both of the inputs are inverted in a NAND latch.

10. The first step of the analysis procedure of SR latch is to ___________
a) label inputs
b) label outputs
c) label states
d) label tables

Answer: b
Explanation: All flip flops have at least one output labeled Q (i.e. inverted). This is so because the flip flops have inverting gates inside them, hence in order to have both Q and Q complement available, we have atleast one output labelled.

11. The inputs of SR latch are ___________
a) x and y
b) a and b
c) s and r
d) j and k
Answer: c
Explanation: SR or Set-Reset latch is the simplest type of bistable multivibrator having two stable states. The inputs of SR latch are s and r while outputs are q and q’. It is clear from the diagram:

12. When a high is applied to the Set line of an SR latch, then ___________
a) Q output goes high
b) Q’ output goes high
c) Q output goes low
d) Both Q and Q’ go high

Answer: a
Explanation: S input of an SR latch is directly connected to the output Q. So when a high is applied Q output goes high and Q’ low.

13. When both inputs of SR latches are low, the latch ___________
a) Q output goes high
b) Q’ output goes high
c) It remains in its previously set or reset state
d) it goes to its next set or reset state

Answer: c
Explanation: When both inputs of SR latches are low, the latch remains in it’s present state. There is no change in output.

14. When both inputs of SR latches are high, the latch goes ___________
a) Unstable
b) Stable
c) Metastable
d) Bistable

Answer: c
Explanation: When both gates are identical and this is “metastable”, and the device will be in an undefined state for an indefinite period.

2.) Flip Flops – 1

1. Latches constructed with NOR and NAND gates tend to remain in the latched condition due to which configuration feature?
a) Low input voltages
b) Synchronous operation
c) Gate impedance
d) Cross coupling
Answer: d
Explanation: Latch is a type of bistable multivibrator having two stable states. Both inputs of a latch are directly connected to the other’s output. Such types of structure is called cross coupling and due to which latches remain in the latched condition.

2. One example of the use of an S-R flip-flop is as ___________
a) Transition pulse generator
b) Racer
c) Switch debouncer
d) Astable oscillator
Answer: c
Explanation: The SR flip-flop is very effective in removing the effects of switch bounce, which is the unwanted noise caused during the switching of electronic devices.

3. The truth table for an S-R flip-flop has how many VALID entries?
a) 1
b) 2
c) 3
d) 4
Answer: c
Explanation: The SR flip-flop actually has three inputs, Set, Reset and its current state. The Invalid or Undefined State occurs at both S and R being at 1.

4. When both inputs of a J-K flip-flop cycle, the output will ___________
a) Be invalid
b) Change
c) Not change
d) Toggle
Answer: c
Explanation: After one cycle the value of each input comes to the same value. Eg: Assume J=0 and K=1. After 1 cycle, it becomes as J=0->1->0(1 cycle complete) and K=1->0->1(1 cycle complete). The J & K flip-flop has 4 stable states: Latch, Reset, Set and Toggle.

5. Which of the following is correct for a gated D-type flip-flop?
a) The Q output is either SET or RESET as soon as the D input goes HIGH or LOW
b) The output complement follows the input when enabled
c) Only one of the inputs can be HIGH at a time
d) The output toggles if one of the inputs is held HIGH

Answer: c
Explanation: In D flip flop, when the clock is high then the output depends on the input otherwise reminds previous output. In a state of clock high, when D is high the output Q also high, if D is ‘0’ then output is also zero. Like SR flip-flop, the D-flip-flop also have an invalid state at both inputs being 1.

6. A basic S-R flip-flop can be constructed by cross-coupling of which basic logic gates?
a) AND or OR gates
b) XOR or XNOR gates
c) NOR or NAND gates
d) AND or NOR gates
Answer: c
Explanation: The basic S-R flip-flop can be constructed by cross coupling of NOR or NAND gates. Cross coupling means the output of second gate is fed to the input of first gate and vice-versa.

7. The logic circuits whose outputs at any instant of time depends only on the present input but also on the past outputs are called ________________
a) Combinational circuits
b) Sequential circuits
c) Latches
d) Flip-flops

Answer: b
Explanation: In sequential circuits, the output signals are fed back to the input side. So, The circuits whose outputs at any instant of time depends only on the present input but also on the past outputs are called sequential circuits. Unlike sequential circuits, if output depends only on the present state, then it’s known as combinational circuits.

8. Whose operations are more faster among the following?
a) Combinational circuits
b) Sequential circuits
c) Latches
d) Flip-flops
Answer: a
Explanation: Combinational circuits are often faster than sequential circuits. Since the combinational circuits do not require memory elements whereas the sequential circuits need memory devices to perform their operations in sequence. Latches and Flip-flops come under sequential circuits.

9. How many types of sequential circuits are?
a) 2
b) 3
c) 4
d) 5
Answer: a
Explanation: There are two type of sequential circuits viz., (i) synchronous or clocked and (ii) asynchronous or unclocked. Synchronous Sequential Circuits are triggered in the presence of a clock signal, whereas, Asynchronous Sequential Circuits function in the absence of a clock signal.

10. The sequential circuit is also called ___________
a) Flip-flop
b) Latch
c) Strobe
d) Adder
Answer: b
Explanation: The sequential circuit is also called a latch because both are a memory cell, which are capable of storing one bit of information.

11. The basic latch consists of ___________
a) Two inverters
b) Two comparators
c) Two amplifiers
d) Two adders

Answer: a
Explanation: The basic latch consists of two inverters. It is in the sense that if the output Q = 0 then the second output Q’ = 1 and vice versa.

12. In S-R flip-flop, if Q = 0 the output is said to be ___________
a) Set
b) Reset
c) Previous state
d) Current state

Answer: b
Explanation: In S-R flip-flop, if Q = 0 the output is said to be reset and set for Q = 1.

13. The output of latches will remain in set/reset untill ___________
a) The trigger pulse is given to change the state
b) Any pulse given to go into previous state
c) They don’t get any pulse more
d) The pulse is edge-triggered
Answer: a
Explanation: The output of latches will remain in set/reset untill the trigger pulse is given to change the state.

14. What is a trigger pulse?
a) A pulse that starts a cycle of operation
b) A pulse that reverses the cycle of operation
c) A pulse that prevents a cycle of operation
d) A pulse that enhances a cycle of operation
Answer: a
Explanation: Trigger pulse is defined as a pulse that starts a cycle of operation.

15. The circuits of NOR based S-R latch classified as asynchronous sequential circuits, why?
a) Because of inverted outputs
b) Because of triggering functionality
c) Because of cross-coupled connection
d) Because of inverted outputs & triggering functionality
Answer: c
Explanation: The cross-coupled connections from the output of one gate to the input of the other gate constitute a feedback path. For this reason, the circuits of NOR based S-R latch classified as asynchronous sequential circuits. Moreover, they are referred to as asynchronous because they function in the absence of a clock pulse.

16. What is an ambiguous condition in a NAND based S’-R’ latch?
a) S’=0, R’=1
b) S’=1, R’=0
c) S’=1, R’=1
d) S’=0, R’=0
Answer: d
Explanation: In a NAND based S-R latch, If S’=0 & R’=0 then both the outputs (i.e. Q & Q’) goes HIGH and this condition is called an ambiguous/forbidden state. This state is also known as an Invalid state as the system goes into an unexpected situation.

17. In a NAND based S’-R’ latch, if S’=1 & R’=1 then the state of the latch is ____________
a) No change
b) Set
c) Reset
d) Forbidden
Answer: a
Explanation: In a NAND based S’-R, latch if S’=1 & R’=1 then there is no any change in the state. It remains in its prior state. This state is used for the storage of data.

18. A NAND based S’-R’ latch can be converted into S-R latch by placing ____________
a) A D latch at each of its input
b) An inverter at each of its input
c) It can never be converted
d) Both a D latch and an inverter at its input
Answer: d
Explanation: A NAND based S’-R’ latch can be converted into S-R latch by placing either a D latch or an inverter at its input as it’s operations will be complementary.

19. One major difference between a NAND based S’-R’ latch & a NOR based S-R latch is ____________
a) The inputs of NOR latch are 0 but 1 for NAND latch
b) The inputs of NOR latch are 1 but 0 for NAND latch
c) The output of NAND latch becomes set if S’=0 & R’=1 and vice versa for NOR latch
d) The output of NOR latch is 1 but 0 for NAND latch
Answer: a
Explanation: Due to inverted input of NAND based S’-R’ latch, the inputs of NOR latch are 0 but 1 for NAND latch.

20. The characteristic equation of S-R latch is ____________
a) Q(n+1) = (S + Q(n))R’
b) Q(n+1) = SR + Q(n)R
c) Q(n+1) = S’R + Q(n)R
d) Q(n+1) = S’R + Q'(n)R
Answer: a
Explanation: A characteristic equation is needed when a specific gate requires a specific output in order to satisfy the truth table. The characteristic equation of S-R latch is Q(n+1) = (S + Q(n))R’.

21. The difference between a flip-flop & latch is ____________
a) Both are same
b) Flip-flop consist of an extra output
c) Latches has one input but flip-flop has two
d) Latch has two inputs but flip-flop has one

Answer: c
Explanation: Flip-flop is a modified version of latch. To determine the changes in states, an additional control input is provided to the latch.

22. How many types of flip-flops are?
a) 2
b) 3
c) 4
d) 5
Answer: c
Explanation: There are 4 types of flip-flops, viz., S-R, J-K, D, and T. D flip-flop is an advanced version of S-R flip-flop, while T flip-flop is an advanced version of J-K flip-flop.

23. The S-R flip flop consist of ____________
a) 4 AND gates
b) Two additional AND gates
c) An additional clock input
d) 3 AND gates
Answer: b
Explanation: The S-R flip flop consists of two additional AND gates at the S and R inputs of S-R latch.

24. What is one disadvantage of an S-R flip-flop?
a) It has no Enable input
b) It has a RACE condition
c) It has no clock input
d) Invalid State

Answer: d
Explanation: The main drawback of s-r flip flop is invalid output when both the inputs are high, which is referred to as Invalid State.

25. One example of the use of an S-R flip-flop is as ____________
a) Racer
b) Stable oscillator
c) Binary storage register
d) Transition pulse generator
Answer: c
Explanation: S-R refers to set-reset. So, it is used to store two values 0 and 1. Hence, it is referred to as binary storage element. It functions as memory storage during the No Change State.

26. When is a flip-flop said to be transparent?
a) When the Q output is opposite the input
b) When the Q output follows the input
c) When you can see through the IC packaging
d) When the Q output is complementary of the input
Answer: b
Explanation: Flip-flop have the property of responding immediately to the changes in its inputs. This property is called transparency.

27. On a positive edge-triggered S-R flip-flop, the outputs reflect the input condition when ________
a) The clock pulse is LOW
b) The clock pulse is HIGH
c) The clock pulse transitions from LOW to HIGH
d) The clock pulse transitions from HIGH to LOW
Answer: c
Explanation: Edge triggered device will follow when there is transition. It is a positive edge triggered when transition takes place from low to high, while, it is negative edge triggered when the transition takes place from high to low.

28. What is the hold condition of a flip-flop?
a) Both S and R inputs activated
b) No active S or R input
c) Only S is active
d) Only R is active

Answer: b
Explanation: The hold condition in a flip-flop is obtained when both of the inputs are LOW. It is the No Change State or Memory Storage state if a flip-flop.

29. If an active-HIGH S-R latch has a 0 on the S input and a 1 on the R input and then the R input goes to 0, the latch will be ________
a) SET
b) RESET
c) Clear
d) Invalid
Answer: b
Explanation: If S=0, R=1, the flip flop is at reset condition. Then at S=0, R=0, there is no change. So, it remains in reset. If S=1, R=0, the flip flop is at the set condition.

30. The circuit that is primarily responsible for certain flip-flops to be designated as edge-triggered is the _____________
a) Edge-detection circuit
b) NOR latch
c) NAND latch
d) Pulse-steering circuit
Answer: a
Explanation: The circuit that is primarily responsible for certain flip-flops to be designated as edge-triggered is the edge-detection circuit.

31. Which circuit is generated from D flip-flop due to addition of an inverter by causing reduction in the number of inputs?
a) Gated JK-latch
b) Gated SR-latch
c) Gated T-latch
d) Gated D-latch
Answer: d
Explanation: Since, both inputs of the D flip-flop are connected through an inverter. And this causes reduction in the number of inputs.

32. The characteristic of J-K flip-flop is similar to _____________
a) S-R flip-flop
b) D flip-flop
c) T flip-flop
d) Gated T flip-flop
Answer: a
Explanation: In an S-R flip-flop, S refers to “SET” whereas R refers to “RESET”. The same behaviour is shown by J-K flip-flop.

33. A J-K flip-flop can be obtained from the clocked S-R flip-flop by augmenting ___________
a) Two AND gates
b) Two NAND gates
c) Two NOT gates
d) Two OR gates

Answer: a
Explanation: A J-K flip-flop can be obtained from the clocked S-R flip-flop by augmenting two AND gates.

34. How is a J-K flip-flop made to toggle?
a) J = 0, K = 0
b) J = 1, K = 0
c) J = 0, K = 1
d) J = 1, K = 1
Answer: d
Explanation: When j=k=1 then the race condition is occurs that means both output wants to be HIGH. Hence, there is toggle condition is occurs, where 0 becomes 1 and 1 becomes 0. That is device is either set or reset.

35. The phenomenon of interpreting unwanted signals on J and K while Cp (clock pulse) is HIGH is called ___________
a) Parity error checking
b) Ones catching
c) Digital discrimination
d) Digital filtering
Answer: b
Explanation: Ones catching means that the input transitioned to a 1 and back very briefly (unintentionally due to a glitch), but the flip-flop responded and latched it in anyway, i.e., it caught the 1. Similarly for 0’s catching.

36. In J-K flip-flop, “no change” condition appears when ___________
a) J = 1, K = 1
b) J = 1, K = 0
c) J = 0, K = 1
d) J = 0, K = 0
Answer: d
Explanation: If J = 0, K = 0, the output remains unchanged. This is the memory storing state.

37. A J-K flip-flop with J = 1 and K = 1 has a 20 kHz clock input. The Q output is ________
a) Constantly LOW
b) Constantly HIGH
c) A 20 kHz square wave
d) A 10 kHz square wave
Answer: d
Explanation: The flip flop is sensitive only to the positive or negative edge of the clock pulse. So, the flip-flop toggles whenever the clock is falling/rising at edge. This triggering of flip-flop during the transition state, is known as Edge-triggered flip-flop. Thus, the output curve has a time period twice that of the clock. Frequency is inversely related to time period and hence frequency gets halved.

38. What is the significance of the J and K terminals on the J-K flip-flop?
a) There is no known significance in their designations
b) The J represents “jump,” which is how the Q output reacts whenever the clock goes high and the J input is also HIGH
c) The letters were chosen in honour of Jack Kilby, the inventory of the integrated circuit
d) All of the other letters of the alphabet are already in use

Answer: c
Explanation: The letters J & K were chosen in honour of Jack Kilby, the inventory of the integrated circuit. In J&K flip-flops, the invalid state problem is resolved, thus leading to the toggling of states.

39. On a J-K flip-flop, when is the flip-flop in a hold condition?
a) J = 0, K = 0
b) J = 1, K = 0
c) J = 0, K = 1
d) J = 1, K = 1
Answer: a
Explanation: At J=0 k=0 output continues to be in the same state. This is the memory storing state.

40. Two J-K flip-flops with their J-K inputs tied HIGH are cascaded to be used as counters. After four input clock pulses, the binary count is ________
a) 00
b) 11
c) 01
d) 10

Answer: a
Explanation: Every O/P repeats after its mod. Here mod is 4 (because 2 flip-flops are used. So mod = 22 = 4). So after 4 clock pulses the O/P repeats i.e. 00.

41. Four J-K flip-flops are cascaded with their J-K inputs tied HIGH. If the input frequency (fin) to the first flip-flop is 32 kHz, the output frequency (fout) is ________
a) 1 kHz
b) 2 kHz
c) 4 kHz
d) 16 kHz

Answer: b
Explanation: 32/2=16:-first flip-flop, 16/2=8:- second flip-flop, 8/2=4:- third flip-flop, 4/2=2:- fourth flip-flop. Since the output frequency is determined on basis of the 4th flip-flop.

42. Determine the output frequency for a frequency division circuit that contains 12 flip-flops with an input clock frequency of 20.48 MHz.
a) 10.24 kHz
b) 5 kHz
c) 30.24 kHz
d) 15 kHz
Answer: b
Explanation: 12 flip flops = 212 = 4096
Input Clock frequency = 20.48*106 = 20480000
Output Clock frequency = 20480000/4096 = 5000 i.e., 5 kHz.

43. How many flip-flops are in the 7475 IC?
a) 2
b) 1
c) 4
d) 8
Answer: c
Explanation: There are 4 flip-flops used in 7475 IC and those are D flip-flops only.